# Evaluation Draft

# 1. Experimental Setup

This simulation recreates a satellite internet network in software. The network uses 720 satellites flying in low Earth orbit (about 1,200 km up), organized into 18 circular paths around the poles with 40 satellites in each path. On the ground, there are 46 gateway stations spread around the world that act as connection hubs. Because there are no inter-satellite links; all traffic routes through gateways, and each gateway has a capacity of 920 Gbps. Totaling a network capacity of 42,320 Gbps. The network graph contains 766 nodes (720 satellites + 46 gateways). A gateway-satellite link exists when the satellite is above 10 degrees elevation from the gateway. This was decided by our team because it isn't stated in the paper explicitly. Our simulation runs 200 routing demands per load level across 13 load levels from 0% to 120% of total capacity on the network. Each demand requests 1 Gbps of bandwidth between a randomly selected source and destination gateway pair. Testing how well the network handles the demand.

# 2. Evaluation Metrics

Four metrics are measured at each load level:

Matrix size: number of nodes in the routing graph used per demand (766 for GORA, variable for AORA).

Computation time: average Dijkstra execution time per demand in milliseconds.

Routing capacity: fraction of demands successfully routed given available link capacity.

Propagation delay: average end-to-end path delay in milliseconds for accepted demands.

# 3. Baselines

Two algorithms from the paper are compared:

GORA (Global Optimal Routing Algorithm): Runs Dijkstra on the full 766-node correlation matrix for every demand. Always finds the globally optimal path but scales with the full network size.

AORA (Approximate Optimal Routing Algorithm): Finds the spherical midpoint between source and destination, selects gateways within a search radius (600 km), builds a reduced sub-matrix from those gateways and their visible satellites, then runs Dijkstra on that. Falls back to GORA if no path is found in the sub-matrix. Faster on average but occasionally suboptimal.

# 4. Preliminary Results

Results below are from the default simulation run (200 demands, 13 load levels, seed 42). Values shown are at approximately 60% traffic load.

| Metric | GORA | AORA |
| --- | --- | --- |
| Matrix size (nodes) | 766 | 303 |
| Avg computation time (ms) | 6.05 | 3.05 |
| Routing capacity (%) | 85.5 | 85.5 |
| Avg propagation delay (ms) | 60.7 | 60.5 |
| Fallback rate (%) | N/A | 28.0 |

AORA sub-matrix sizes ranged from approximately 191 to 356 nodes across all load levels, compared to GORA's fixed 766 nodes. AORA computation time was approximately 2x faster than GORA at low load levels. Both algorithms achieved 100% routing capacity only at 0% load, dropping to roughly 77-90% from 10% load onward and staying roughly flat through 120% load. GORA and AORA matched each other exactly at every load level, meaning AORA's smaller search space did not cause it to miss any paths that GORA found.